Vivado HLS Design Flow Lab

Introduction

This lab provides a basic introduction to high-level synthesis using the Vivado HLS tool flow.  You will use Vivado HLS in GUI mode to create a project. You will simulate, synthesize, and implement the provided design.

Objectives

After completing this lab, you will be able to:

Create a new project using Vivado HLS GUI

Simulate a design

Synthesize a design

Implement a design

Perform design analysis using the Analysis capability of Vivado HLS

Analyze simulator output using Vivado and XSim simulator

Procedure

This lab is separated into steps that consist of general overview statements that provide information on the detailed instructions that follow. Follow these detailed instructions to progress through the lab.

This lab comprises 8 primary steps: You will create a new project in Vivado HLS, run simulation, run debug, synthesize the design, open an analysis perspective, run RTL co-simulation, view simulation results using Vivado and XSim, and export and implement the design in Vivado HLS.

General Flow for this Lab

Step 5: Analyze using Analysis Perspective

Step 1: Creating a New 
Project

Step 2:
Run C Simulation

Step 3:
Run Debugger

Step 4: Synthesize the design

Step 6: 
Run C/RTL  Co-Simulation

Step 7: Viewing Simulation Results in Vivado

Step 8:

Export RTL and Implement

Create a New Project	Step 1

Create a new project in Vivado HLS targeting Zynq xc7z020clg484-1 (ZedBoard) or xc7z010clg400-1 (Zybo).

Launch Vivado HLS: Select Start > All Programs > Xilinx Design Tools > Vivado 2015.4  > Vivado HLS > Vivado HLS 2015.4

A Getting Started GUI will appear.

Figure 1. Getting Started view of Vivado-HLS

In the Getting Started GUI, click on Create New Project. The New Vivado HLS Project wizard opens.

Click the Browse… button of the Location field and browse to c:\xup\hls\labs\lab1 and then click OK.

For Project Name, type matrixmul.prj

Figure 2. New Vivado HLS Project wizard

Click Next.

In the Add/Remove Files window, type matrixmul as the Top Function name (the provided source file contains the function, to be synthesized, called matrixmul).

Click the Add Files… button, select matrixmul.cpp file from the c:\xup\hls\labs\lab1 folder, and then click Open.

Click Next.

In the Add/Remove Files for the testbench, click the Add Files… button, select matrixmul_test.cpp file from the c:\xup\hls\labs\lab1 folder and click Open.

Select the matrixmul_test.cpp in the files list window and click the Edit CFLAG…  button, type 
-DHW_COSIM, and click OK. (This defines a custom flag that will be used later.)

Click Next.

In the Solution Configuration page, leave Solution Name field as solution1 and set the clock period as 10 (for ZedBoard) or 8 (for Zybo).  Leave Uncertainty field blank it will take 1.25 as the default value for ZedBoard and 1 for Zybo.

Click the … button in the Part Selection section.

In the Device Selection Dialog page, select Parts Specify field, and select the following filters to select the xc7z020clg484-1 (ZedBoard) or xc7z010clg400-1 (Zybo)  part, and click OK:

Family: Zynq

Sub-Family: Zynq

Package: clg484 (for ZedBoard) or clg400 (for Zybo)

Speed Grade: –1

Figure 3. Using Parts Specify option in Part Selection Dialog

You can also select the Boards specify option (only for ZedBoard) and select one of the listed board if the desired target board is listed.

Figure 4. Using Boards Specify option in Part Selection Dialog

Click Finish.

You will see the created project in the Explorer view.  Expand various sub-folders to see the entries under each sub-folder.

Figure 5. Explorer Window

Double-click on the matrixmul.cpp under the source folder to open its content in the information pane.

Figure 6. The Design under consideration

It can be seen that the design is a matrix multiplication implementation, consisting of three nested loops.  The Product loop is the inner most loop performing the actual Matrix elements product and sum.  The Col loop is the outer-loop which feeds the next column element data with the passed row element data to the Product loop.  Finally, Row is the outer-most loop.  The res[i][j]=0 (line 79) resets the result every time a new row element is passed and new column element is used.

Run C Simulation	Step 2

Run C simulation to view the expected output.

Select Project > Run C Simulation or click on  from the tools bar buttons, and Click OK in the C Simulation Dialog window.

The files will be compiled and you will see the output in the Console window.

Figure 7. Program output

Double-click on matrixmul_test.cpp under testbench folder in the Explorer to see the content.

You should see two input matrices initialized with some values and then the code that executes the algorithm. If HW_COSIM is defined (as was done during the project set-up) then the matrixmul function is called and compares the output of the computed result with the one returned from the called function, and prints Test passed if the results match.

If HW_COSIM had not been defined then it will simply output the computed result and not call the matrixmul function.

Run Debugger	Step 3

Run the application in debugger mode and understand the behavior of the program.

Select Project > Run C Simulation or click on  from the tools bar buttons.  Select the Launch Debugger option and click OK.

The application will be compiled with –g option to include the debugging information, the compiled application will be invoked, and the debug perspective will be opened automatically.

The Debug perspective will show the matrixmul_test.cpp in the source view, argc and argv variables defined in the Variables view, Outline view showing the objects which are in the current scope, thread created and the program suspended at the main() function entry point.

Figure 8. A Debug perspective

Scroll-down in the source view, and double-click in the blue margin at line 105 where it is about to output “{“ in the output console window.  This will set a break-point at line 105.  .

The breakpoint is marked with a blue circle, and a tick

Similarly, set a breakpoint at line 101 on the matrixmul() function

Using the Step Over (F6) button () several times, observe the execution progress, and observe the variable values updating, as well as computed software result.

Figure 9. Debugger’s intermediate output view

Now click the Resume () button or F8 to complete the software computation and stop at line 101.

Observe the following computed software result in the variables view.

Figure 10. Software computed result

Click on the Step Into (F5) button () to traverse into the matrixmul module, the one that we will synthesize, and observe that the execution is paused on line 75 of the module.

Using the Step Over (F6) several times, observe the computed results.  Once satisfied, you can use the Step Return (F7) button to return from the function.

The program execution will suspend at line 105 as we had set a breakpoint.  Observe the software and hardware (function) computed results in the Variables view.

Figure 11. Computed results

Set a breakpoint on line 134 (return err_cnt;), and click on the Resume button.

The execution will continue until the breakpoint is encountered.  The console window will show the results as seen earlier (Figure 7).

Press the Resume button or Terminate button to finish the debugging session.

Synthesize the Design	Step 4

Switch to Synthesis view and synthesize the design with the defaults.  View the synthesis results and answer the question listed in the detailed section of this step.

Switch to the Synthesis view by clicking  on the tools bar.

Select Solution > Run C Synthesis > Active Solution or click on the  button to start the synthesis process.

When synthesis is completed, the Synthesis Results will be displayed along with the Outline pane. Using the Outline pane, one can navigate to any part of the report with a simple click.

Figure 12. Report view after synthesis is completed

If you expand solution1 in Explorer, several generated files including report files will become accessible.

Figure 13. Explorer view after the synthesis process

Note that when the syn folder under the Solution1 folder is expanded in the Explorer view, it will show report, systemC, verilog, and vhdl sub-folders under which report files, and generated source (vhdl, verilog, header, and cpp) files.  By double-clicking any of these entries will open the corresponding file in the information pane.

Also note that if the target design has hierarchical functions, reports corresponding to lower-level functions are also created.

The Synthesis Report shows the performance and resource estimates as well as estimated latency in the design.

Using scroll bar on the right, scroll down into the report and answer the following question.

Question 1

Estimated clock period:							
Worst case latency:							
Number of DSP48E used:						
Number of FFs used:							
Number of LUTs used:

The report also shows the top-level interface signals generated by the tools.

Figure 14. Generated interface signals

You can see ap_clk, ap_rst and ap_ idle and ap_ready control signals are automatically added to the design by default.  These signals are used as handshaking signals to indicate when the design is ready to begin the next computation command (ap_ready), when the next computation is started (ap_start), and when the computation is completed (ap_done).  Other signals are generated based on the input and output signals in the design and their default or specified interfaces.

Analyze using Analysis Perspective	Step 5

Switch to the Analysis Perspective and understand the design behavior.

Select Solution > Open Analysis Perspective or click on () to open the analysis viewer.

The Analysis perspective consists of 5 panes as shown below. Note that the module and loops hierarchies are displayed unexpanded by default.

The Module Hierarchy pane shows both the performance and area information for the entire design and can be used to navigate through the hierarchy. The Performance Profile pane is visible and shows the performance details for this level of hierarchy. The information in these two panes is similar to the information reviewed earlier in the synthesis report.

The Performance view is also shown in the right-hand side pane. This view shows how the operations in this particular block are scheduled into clock cycles.

The left-hand column lists the resources

The top row lists the control states (c0 to c5) in the design. Control states are the internal states used by High-Level Synthesis to schedule operations into clock cycles. There is a close correlation between the control states and the final states in the RTL Finite State Machine(FSM) but there is no one-to-one mapping

Figure 15. Analysis perspective

Click on loop Row to expand, and then click on sub-loops Col and Product to fully expand the loop hierarchy.

Figure 16. Performance matrix showing top-level Row operation

From this we can see that in the first state (C1) of the Row the loop exit condition is checked and there is an add operation performed. This addition is likely the counter to count the loop iterations, and we can confirm this.

The operations resulting from the loops are colored yellow, the standard operations are colored purple, and sub-blocks will be colored green (in our case we don’t have any lower-level functions).

Select the purple block for the adder in state C1, right-click and select Goto Source.

The source code pane will be opened, highlighting line 75 where the Row loop index is being tested and incremented. In the next state (C2) it starts to execute the Col loop.

Figure 17. Cross probing into the source file

In C2, click on the purple blocks for the operations (e.g. p_addr8) in the Col loop to see the source code highlighting (line 79) update.

Expand the Performance Profile hierarchy and note iteration latencies, Trip counts, and overall latencies for each of the nested loops.

Figure 18. The Performance Profile output

The number of iterations can also be noted by holding the mouse over the loop in the Performance view (a dialog box shows the loop statistics).

Figure 19. Loop information

Note that the initiation interval does not have a number as this loop is not pipelined.

Click next to the matrixmul entry in the Module Hierarchy and observe that the entry is not expanded, since there are no lower-level functions defined in the design.

Select the Resource Profile tab and observe various resources and where they have been used. You can expand Expressions and Registers sections to see how the resources are being used by which operations.

Figure 20. The Resource Profile tab view

In the Performance Matrix tab, select the Resource tab (at the bottom of the page), and expand Expressions, I/O Ports, and Memory Ports entries to view the type of operations, resources used, and in which state they are being used.

Figure 21. The Resource tab

Click on the Synthesis tool bar button to switch back to the Synthesis view.

Run C/RTL Co-simulation	Step 6

Run the C/RTL Co-simulation with the default settings of VHDL.  Verify that the simulation passes.

Select Solution > Run C/RTL Cosimulation or if you are in the synthesis view, click on the   toolbar button to open the dialog box so the desired simulations can be selected and run.

A C/RTL Co-simulation Dialog box will open.

Make sure the VHDL option is selected.

This allows the simulation to be performed using VHDL. To perform the verification using Verilog, you can select Verilog and choose the simulator from the drop-down menu or let the tools use the first simulator that appears in the PATH variable.

Figure 22. A C/RTL Co-simulation Dialog

Click OK to run the VHDL simulation.

The C/RTL Co-simulation will run, generating and compiling several files, and then simulating the design.  It goes through three stages.

- First, the VHDL test bench is executed to generate input stimuli for the RTL design

- Second, an RTL test bench with newly generated input stimuli is created and the RTL simulation is then performed

- Finally, the output from the RTL is re-applied to the VHDL test bench to check the results

In the console window you can see the progress and also a message that the test is passed.  This eliminates writing a separate testbench for the synthesized design.

Figure 23. Console view showing simulation progress

Once the simulation verification is completed, the simulation report tab will open showing the results. The report indicates if the simulation passed or failed. In addition, the report indicates the measured latency and interval.

Since we have selected only VHDL, the result shows the latencies and interval (initiation) which indicates after how many clock cycles later the next input can be provided.  Since the design is not pipelined, it will be latency+1 clock cycles.

- ZedBoard					(b) ZYBO

Figure 24. Co-simulation results

Viewing Simulation Results in Vivado	Step 7

Run Verilog simulation with Dump Trace option selected.

Select Solution > Run C/RTL Cosimulation or click on the   button in the Synthesis view to open the dialog box so the desired simulations can be run.

Click on the Verilog RTL Selection option, leaving Verilog/VHDL Simulator Section option to Auto.

Optionally, you can click on the drop-down button and select the desired simulator from the available list of XSim, ISim, ModelSim, and Riviera.

Select All for the Dump Trace option and click OK.

Figure 25. Setting up for Verilog simulation and dump trace

When RTL verification completes the co-simulation report automatically opens showing the Verilog simulation has passed (and the measured latency and interval). In addition, because the Dump Trace option was used and Verilog was selected, two trace files entries can be seen in the Verilog simulation directory

Figure 26. Explorer view after the Verilog RTL co-simulation run

The Cosimulation report shows the test was passed for Verilog along with latency and Interval results.  It also shows the SystemC results of the previous run.

- ZedBoard					(b) ZYBO

Figure 27. Cosimulation report

Start Vivado 2015.4 and enter Tcl commands to open and view the dumped traces.

Select Start > All Programs > Xilinx Design Tools > Vivado 2015.4  > Vivado 2015.4 to start the Vivado Design Suite program.

In the Vivado Tcl console, enter the following commands one by one:

cd c:/xup/hls/labs/lab1/matrixmul.prj/solution1/sim/Verilog
current_fileset
open_wave_database matrixmul.wdb
create_wave_config 
add_wave /

The above commands will load the project, simulation results, and open the waveform.

In the waveform window, click on the full zoom tool button ( ) to see the entire simulation of one iteration.

Select a_address0 in the waveform window, right-click and select Radix > Unsigned Decimal.  Similarly, do the same for b_address0 and res_address0 signals.

Similarly, set the a_q0, b_q0, and res_d0 radix to Signed Decimal.

Scroll the waveform little, so you can view the main interface signals (ap_*).

Figure 28. Full waveform showing iteration worth simulation

Note that as soon as ap_start is asserted, ap_idle has been de-asserted indicating that the design is in computation mode.  The ap_idle signal remains de-asserted until ap_done is asserted, indicating completion of the process.  This indicates 106 clock cycles latency.

Using the Zoom In button, view area of ~120 ns and ~550 ns.

Figure 29.  Zoomed view

Observe that the design expects element data by providing a_address0, a_ceo, b_address0, b_ceo signals and outputs result using res_d0, res_we0, and res_ce0.

View various part of the simulation and try to understand how the design works.

When done, close Vivado by selecting File > Exit. Click OK if prompted, and then Discard to close the program without saving.

Export RTL and Implement	Step 8

In Vivado HLS, export the design, selecting VHDL as a language, and run the implementation by selecting Evaluate option.

In Vivado-HLS, select Solution > Export RTL or click on the  button to open the dialog box so the desired implementation can be run.

An Export RTL Dialog box will open.

Figure 30. A Export RTL Dialog box

With default settings (shown above), the IP packaging process will run and create a package for the Vivado IP Catalog. Other options, available from the drop-down menu, are to create IP packages for System Generator for DSP/System Generator for DSP using ISE, create a pcore for Xilinx Platform Studio, or create a Synthesized checkpoint.

Click on the drop-down menu of the Options field, and select VHDL and click on the Evaluate check box as the preferred language and to run the implementation tool.

Click OK and the implementation run will begin.

You can observe the progress in the Vivado HLS Console window.  It goes through several phases:

Exporting RTL as an IP in the IP-XACT format

RTL evaluation, since we selected Evaluate option

  - Goes through Synthesis

  - Goes through Placement and Routing

Figure 31. Console view

When the run is completed the implementation report will be displayed in the information pane.

- ZedBoard					(b) ZYBO

Figure 32. Implementation results in Vivado HLS (Zedboard and Zybo)

Observe that the timing constraint was met, the achieved period (7.522 [ZedBoard], 5.943 [Zybo] ns), and the type and amount of resources used.

Collapse the Explorer view and observe that impl folder is created under which ip, report, Verilog, and vhdl sub-folders are created.

Figure 33. Explorer view after the RTL Export run

Expand the Verilog and vhdl sub-folders and observe that the Verilog sub-folder only has the rtl file whereas the vhdl sub-folder has several files and sub-folders as the synthesis and implementation runs were made for it.

It includes project.xpr file (the Vivado project file), matrixmul.xdc file (timing constraint file), project.runs folder (which includes synth_1 and impl_1 sub-folders created by the synthesis and implementation runs) among others.

Figure 34. The implementation directory

Expand the ip folder and observe the IP packaged as a zip file (xilinx_com_hls_matrixmul_1_0.zip), ready for adding to the Vivado IP catalog.

Figure 35. The ip folder content

Close Vivado HLS by selecting File > Exit.

Conclusion

In this lab, you completed the major steps of the high-level synthesis design flow using Vivado HLS. You created a project, adding source files, synthesized the design, simulated the design, and implemented the design. You also learned how to use the Analysis capability to understand the scheduling and binding.

Answers

Answer the following questions:

Estimated clock period:			8.34 ns (ZedBoard) / 6.38 ns

Worst case latency:				106 clock cycles (ZedBoard) / 133 clock cycles (ZYBO)

Number of DSP48E used:			1

Number of FFs used:			61 (ZedBoard) / 78 (ZYBO)

Number of LUTs used:			68 (ZedBoard) / 69 (ZYBO)