Creating a Processor System Lab

Introduction

This lab introduces a design flow to generate a IP-XACT adapter from a design using Vivado HLS and using the generated IP-XACT adapter in a processor system using IP Integrator in Vivado.

Objectives

After completing this lab, you will be able to:

Understand the steps and directives involved in creating an IP-XACT adapter from a synthesized design in Vivado HLS

Create a processor system using IP Integrator in Vivado

Integrate the generated IP-XACT adapter into the created processor system

# The Design

The design consists of a FIR filter to filter a 4 KHz tone added to CD quality (48 KHz) music. The characteristic of the filter is as follows:

FS=48000 Hz

FPASS1=2000 Hz

FSTOP1=3800 Hz

FSTOP2=4200 Hz

FPASS2=6000 Hz

APASS1=APASS2=1 dB

ASTOP=60 dB

This lab requires you to develop a peripheral core of the designed filter that can be instantiated in a processor system.  The processor system will acquire a stereo music stream using an on-board CODEC chip and I2C controller, process it through the designed filter (bandstop filter), and output back to the headphone.

# Procedure

This lab is separated into steps that consist of general overview statements that provide information on the detailed instructions that follow. Follow these detailed instructions to progress through the lab.

This lab comprises 9 primary steps: You will create a new project in Vivado HLS, run simulation, synthesize the design, run RTL/C co-simulation, create a project in Project Navigator, run simulation using ISIM, setup for IP-XACT adapter in Vivado HLS, implement the design in Vivado HLS, create a processor system in Vivado using IP Integrator, create a software application in SDK, and verify the design in hardware.

# General Flow for this Lab

Step 1: Create a New 
Project

Step 2:

Run C

Simulation

Step 3:

Synthesize

the Design

Step 4:

Run RTL/C Co-Simulation

Step 5:

Setup IP-XACT Adapter

Appendix: Create an Initial Design using Tcl Script

Step 9: Verify the Design in Hardware

Step 8: Export to SDK & Create an Application

Step 7:

Create a Vivado Project

Step 6:

Generate the IP-XACT Adapter

- Create a New Project	Step 1

Create a new project in Vivado HLS targeting XC7Z020CLG484-1 (ZedBoard) or XC7Z010CLG400-1 (Zybo).

Launch Vivado HLS: Select Start > All Programs > Xilinx Design Tools > Vivado 2015.4  > Vivado HLS > Vivado HLS 2015.4

A Getting Started GUI will appear.

In the Getting Started section, click on Create New Project. The New Vivado HLS Project wizard opens.

Click Browse… button of the Location field, browse to c:\xup\hls\labs\lab4, and then click OK.

For Project Name, type fir.prj

Click Next.

In the Add/Remove Files for the source files, type fir as the function name (the provided source file contains the function, to be synthesized, called fir).

Click the Add Files… button, select fir.c and fir_coef.dat files from the c:\xup\hls\labs\lab4 folder, and then click Open.

Click Next.

In the Add/Remove Files for the testbench, click the Add Files… button, select fir_test.c file from the c:\xup\hls\labs\lab4 folder and click Open.

Click Next.

In the Solution Configuration page, leave Solution Name field as solution1 and set the clock period as 10 (for ZedBoard) or 8 (for Zybo).  Leave Uncertainty field blank as it will take 1.25 as the default value for ZedBoard and 1 for Zybo.

Click on Part’s Browse button, and select the following filters, using the Parts Specify option, to select xc7z020clg484-1 (ZedBoard) or xc7z010clg400-1 (Zybo), and click OK:

Family: Zynq

Sub-Family: Zynq

Package: clg484 (ZedBoard) or clg400 (Zybo)

Speed Grade: –1

Click Finish.

You will see the created project in the Explorer view.  Expand various sub-folders to see the entries under each sub-folder.

Double-click on the fir.c under the source folder to open its content in the information pane.

Figure 1. The design under consideration

The FIR filter expects x as a sample input and pointer to the computed sample out.  Both of them are defined of data type data_t.  The coefficients are loaded in array c of type coef_t from the file called fir_coef.dat located in the current directory. The sequential algorithm is applied and accumulated value (sample out) is computed in variable acc of type acc_t.

Double-click on the fir.h in the outline tab to open its content in the information pane.

Figure 2.  The header file

The header file includes ap_cint.h so user defined data width (of arbitrary precision) can be used.  It also defines number of taps (N), number of samples to be generated (in the testbench), and data types coef_t, data_t, and acc_t.  The coef_t and data_t are short (16 bits).  Since the algorithm iterates (multiply and accumulate) over 59 taps, there is a possibility of bit growth of 6 bits and hence acc_t is defined as int38.  Since the acc_t is bigger than sample and coefficient width, they have to cast before being used (like in lines 16, 18, and 21 of fir.c).

Double-click on the fir_test.c under the testbench folder to open its content in the information pane.

Notice that the testbench opens fir_impulse.dat in write mode, and sends an impulse (first sample being 0x8000.

Run C Simulation	Step 2

Run C simulation to observe the expected output.

Select Project > Run C Simulation or click on  from the tools bar buttons, and Click OK in the C Simulation Dialog window.

The testbench will be compiled using apcc compiler and csim.exe file will be generated. The csim.exe will then be executed and the output will be displayed in the console view.

Figure 3. Initial part of the generated output in the Console view

You should see the filter coefficients being computed.

Synthesize the Design	Step 3

Synthesize the design with the defaults.  View the synthesis results and answer the question listed in the detailed section of this step.

Select Solution > Run C Synthesis > Active Solution to start the synthesis process.

When synthesis is completed, several report files will become accessible and the Synthesis Results will be displayed in the information pane.

The Synthesis Report shows the performance and resource estimates as well as estimated latency in the design.

Using scroll bar on the right, scroll down into the report and answer the following question.

Question 1

Estimated clock period:

Worst case latency:

Number of DSP48E used:

Number of BRAMs used:

Number of FFs used:

Number of LUTs used:

The report also shows the top-level interface signals generated by the tools.

Figure 4. Generated interface signals

You can see the design expects x input as 16-bit scalar and outputs y via pointer of the 16-bit data.  It also has ap_vld signal to indicate when the result is valid.

Add PIPELINE directive to loop and re-synthesize the design.  View the synthesis results.

Make sure that the fir.c is open in the information view.

Select the Directive tab, and apply the PIPELINE directive to the loop.

Select Solution > Run C Synthesis > Active Solution to start the synthesis process.

When synthesis is completed, the Synthesis Results will be displayed in the information pane.

Note that the latency has reduced to 63 clock cycles.  The DSP48 and BRAM consumption remains same; however, LUT and FF consumptions have slightly increased.

Run RTL/C CoSimulation	Step 4

Run the RTL/C Co-simulation, selecting Verilog.  Verify that the simulation passes.

Select Solution  Run C/RTL Cosimulation or click on the   button to open the dialog box so the desired simulations can be run.

A C/RTL Co-simulation Dialog box will open.

Select the Verilog option and click OK.

The Co-simulation will run, generating and compiling several files, and then simulating the design.  In the console window you can see the progress.  When done the RTL Simulation Report shows that it was successful and the latency reported was 63.

Setup IP-XACT Adapter	Step 5

Add INTERFACE directive to create AXI4LiteS adapters so IP-XACT adapter can be generated during the RTL Export step.

Make sure that fir.c file is open and in focus in the information view.

Select the Directive tab.

Right-click x, and click on Insert Directive….

In the Vivado HLS Directive Editor dialog box, select INTERFACE using the drop-down button.

Click on the button beside mode (optional).  Select s_axilite.

In the bundle (optional) field, enter fir_io and click OK.

Figure 5. Selecting the AXI4LiteS adapter and naming bundle

Similarly, apply the INTERFACE directive (including bundle) to the y output.

Figure 6. Applying bundle to assign y output to AXI4Lite adapter

Apply the INTERFACE directive to the top-level module fir to include ap_start, ap_done, and ap_idle signals as part of bus adapter (the variable name shown will be return).  Include the bundle information too.

Figure 7. Applying bundle to assign function control signals to AXI4Lite adapter

Note that the above steps 5-1-3 through 5-1-8 will create address maps for x, y, ap_start ap_valid, ap_done, and ap_idle, which can be accessed via software. Alternately, ap_start, ap_valid, ap_done, ap_idle signals can be generated as separate ports on the core by not applying RESOURCE directive to the top-level module fir. These ports will then have to be connected in a processor system using available GPIO IP.

Generate IP-XACT Adapter 	Step 6

Re-synthesize the design as directives have been added.  Run the RTL Export to generate the IP-XACT adapter.

Since the directives have been added, it is safe to re-synthesize the design. Select Solution > Run C Synthesis > Active Solution

Check the Interface summary at the bottom of the Synthesis report to see the interface that has been created.

Once the design is synthesized, select Solution > Export RTL to open the dialog box so the desired IP can be generated.

An Export RTL Dialog box will open.

Figure 8. Export RTL Dialog

Click OK to generate the IP-XACT adapter.

When the run is completed, expand the impl folder in the Explorer view and observe various generated directories; ip, verilog and vhdl.

Figure 9. IP-XACT adapter generated

Expand the ip directory and observe several files and sub-directories.  One of the sub-directory of interest is the drivers directory which consists of header, c, tcl, mdd, and makefile files. Another file of interest is the zip file, which is the ip repository file that can be imported in an IP Integrator design

Figure 10. Adapter’s drivers directory

Close Vivado HLS by selecting File > Exit.

Create a Vivado Project 	Step 7

Launch Vivado Tcl Shell and run the provided tcl script to create an initial system targeting either the Zedboard (having xc7z020clg484-1 device) or Zybo (having xc7z010clg400-1 device).

If you want to create the system from scratch then follow the steps provided in Appendix and then continue from step 7-2 below.

Open Vivado Tcl Shell by selecting Start > All Programs > Xilinx Design Tools > Vivado 2015.4 > Vivado 2015.4 Tcl Shell

In the shell window, change the directory to c:/xup/hls/labs/lab4 using the cd account.

Run the provided script file to create an initial system having zed_audio_ctrl and GPIO peripherals by typing the following command:

source zed_audio_project_create.tcl for ZedBoard or

source zybo_audio_project_create.tcl for Zybo

The script will be run and the initial system, shown below, will be created.

Figure 11. Block design having zed_audio_ctrl and connections made for ZedBoard

Figure 11. Block design having zybo_audio_ctrl and connections made for Zybo

Add the HLS IP to the IP Catalog

In the Flow Navigator pane, click Project Settings under Project Manager.

Click the IP icon.

Click the + button (The lab4/ip_repo directory has already been added). Browse to c:\xup\hls\labs\lab4\fir.prj\solution1\impl\ip and click Select.

The directory will be scanned and added in the IP Repositories window, and one IP entry will be detected.

Click OK.

Figure 12. Setting path to IP Repositories

Click OK to accept the settings.

Instantiate fir_top core twice, one for each side channel, into the processing system naming the instances as fir_left and fir_right.

Click the Add IP icon  and search for Fir in the catalog by typing Fir and double-click on the Fir entry to add an instance.

Click on the Add IP to Block Design button if presented.

Notice that the added IP has HLS logo in it indicating that this was created by Vivado HLS.

Select the added instance in the diagram, and change its instance name to fir_left by typing it in the Name field of the Block Properties form in the left.

Similarly, add another instance of the HLS IP, and name it fir_right.

Click on Run Connection Automation, and select All Automation

Click on /fir_left/s_axi_fir_io, and /fir_right/ s_axi_fir_io and verify that they will both be connected to the M_AXI_GP0, and click OK.

Similarly, click on Run Connection Automation again, and select /fir_right/ s_axi_fir_io and click OK.

Enable the PS-PL Interrupt ports > IRQ_F2P ports. Add an instance of concat IP with two single-bit input ports.  Connect input ports to the interrupt ports of the two FIR instances and the output port to the IRQ_F2P port of the processing_system7_0 instance.

Double-click on the processing_system7_0 instance to open the re-customization form.

Select the Interrupt in the left pane, click on the Fabric Interrupts check box in the right.

Expand the Fabric Interrupts > PL-PS Interrupt Ports > IRQ_F2P entry in the right, and click the check-box of IRQ_F2P[15:0].

Click OK.

Add an instance of the concat IP.

Connect the interrupt port of each of the FIR instances to the two input ports of the xlconcat_0 instance.

Connect the output port of the xlconcat_0 instance to the IRQ_F2P port of the processing_system7_0 instance.

At this stage the design should look like shown below (you may have to click the regenerate button).

(a) ZedBoard

(b) Zybo

Figure 13. The complete hardware design

Verify addresses and validate the design. Generate the system_wrapper file, and add the provided Xilinx Design Contraints (XDC).

Click on the Address Editor, and expand the processing_system7_0 > Data if necessary.

The generated address map should look like as shown below.

(a) ZedBoard

(b) Zybo

Figure 14. Generated address map

Run Design Validation (Tools > Validate Design) and verify there are no errors

In the sources view, right-click on the block diagram file, system.bd, and select Create HDL Wrapper to update the HDL wrapper file. When prompted, click OK with the Let Vivado manage wrapper and auto-update option.

Click Add Sources in the Flow Navigator pane, select Add or Create Constraints, and click Next.

Click the Add Files button, browse to the c:\xup\hls\labs\lab4 folder, select zed_audio_constraints.xdc or zybo_audio_constraints.xdc

Click Finish to add the file.

Click on the Generate Bitstream in the Flow Navigator to run the synthesis, implementation, and bitstream generation processes.

Click Save and Yes if prompted to start the process.

When the bit generation is completed, a selection box will be displayed with Open Implemented Design option selected. Click Cancel.

Export to SDK and create Application Project	Step 8

Export the hardware along with the generated bitstream to SDK.

Select File > Export > Export Hardware…

Make sure that Include Bitstream option is selected and click OK, leaving the target directory set to local project directory.

Select File > Launch SDK

Click OK.

In SDK, select File > New > Board Support Package.

Click Finish with the default settings (with standalone operating system).

This will open the Software Platform Settings form showing the OS and libraries selections.

Click OK to accept the default settings, as we want to create a standalone_bsp_0 software platform project without requiring any additional libraries support.

The library generator will run in the background and will create xparameters.h file in the C:\xup\hls\labs\lab4\audio\audio.sdk\standalone_bsp_0\ps7_cortexa9_0\include\ directory.

Select File > New > Application Project.

Enter TestApp as the Project Name, and for Board Support Package, choose Use Existing (standalone_bsp should be the only option)

Click Next, and select Empty Application and click Finish

Select TestApp in the project view, right-click the src folder, and select Import.

Expand General category and double-click on File System.

Browse to c:\xup\hls\labs\lab4 folder and click OK

Select both zed_testapp.c and zed_audio.h for ZedBoard or zybo_testapp.c and zybo_audio.h for Zybo and click Finish to add the file to the project.

The program should compile successfully.

Verify the Design in Hardware	Step 9

Zybo: Make sure that the JP7 is set to select USB power.

- Connect a micro-usb cable between  a PC and the JTAG port of the board. Connect an audio patch cable between the Line In jack and the speaker (headphone) out jack of a PC. Connect a headphone to the Line Out jack (ZedBoard) or HPH OUT (Zybo) on the board. Power ON the board.

Zybo only: Make sure that the JP7 is set to select USB power.

Connect a micro-usb cable between a PC and the JTAG port of the board.

Connect an audio patch cable between the Line In jack and the speaker (headphone) out jack of a PC.

Connect a headphone to the Line Out jack on ZedBoard or HPH Out jack on Zybo board. Power ON the board.

Select Xilinx Tools > Program FPGA.

Make sure that the system_wrapper.bit bitstream is selected and the BMM file field is blank.

Click Program.

This will configure the FPGA.

Double-click corrupted_music_4KHz.wav or some other wave file of interest to play it using the installed media player.  Place it in the continuous play mode.

Right-click on the TestApp in the Project Explorer pane and select Run As > Launch On Hardware (GDB).

The program will be downloaded and run.  If you want to listen to corrupted signal then set the SW0 OFF. To listened the filtered signal set the SW0 ON.

When done, terminate the program by clicking the Terminate (Red square) button in the Console tab of the SDK. power OFF the board, and exit SDK and Vivado using File > Exit.

Conclusion

In this lab, you added RESOURCE directive to create an IP-XACT adapter.  You generated the IP-XACT adapter during the implementation phase.  You then created a processor system using IP Integrator, integrated the generated IP-XACT adapter, and tested the system with the provided application.

Answers

Answer the following questions:

Estimated clock period:			6.38 ns

Worst case latency:				175 clock cycles

Number of DSP48E used:			3

Number of BRAMs used:			0

Number of FFs used:			168

Number of LUTs used:			106

Appendix

Create a Project using Vivado GUI	Step 10

Launch Vivado and create an empty project targeting the Zedboard (having xc7z020clg484-1 device) or Zybo (having xc7z010clg400-1 device)  and using the Verilog language.

Open Vivado by selecting Start > All Programs > Xilinx Design Tools > Vivado 2015.4 > Vivado 2015.4

Click Create New Project to start the wizard. You will see the Create a New Vivado Project dialog box. Click Next.

Click the Browse button of the Project Location field of the New Project form, browse to c:\xup\hls\labs\lab4, and click Select.

Enter audio in the Project Name field.  Make sure that the Create Project Subdirectory box is checked.  Click Next.

Figure A-1. Project Name entry

Select RTL Project in the Project Type form, and click Next.

Select Verilog as the Target language and Simulator Language in the Add Sources form, and click Next.

Figure A-2. Add sources to new project

Click Next two times to skip Adding Existing IP and Add Constraints dialog boxes

In the Default Part form, select Boards, and either select Zedboard Zynq Evaluation and Development Kit or Zybo. Click Next.

If you don’t see Zybo entry and wants to target Zybo board then please read readme_zybo.docx file and install the zybo board files in the Vivado installation directory.

Figure A-3. Boards and Parts selection

Check the Project Summary and click Finish to create an empty Vivado project.

Creating the System Using the IP Integrator	Step 11

Use the IP Integrator to create a new Block Design, and generate the ARM Cortex-A9 processor based hardware system.

In the Flow Navigator, click Create Block Design under IP Integrator

Figure A-4. Create IP Integrator Block Diagram

Enter system for the design name and click OK

IP from the catalog can be added in different ways. Click on Add IP in the message at the top of the Diagram panel, or click the Add IP icon  in the block diagram side bar, press Ctrl + I,  or right-click anywhere in the Diagram workspace and select Add IP.

Once the IP Catalog is open, type “zy” into the Search bar, find and double click on ZYNQ7 Processing System entry, or click on the entry and hit the Enter key to add it to the design.

The Zynq block will be added.

Figure A-5. The Zynq IP Block

Notice the message at the top of the Diagram window that Designer Assistance available. Click on Run Block Automation and select /processing_system7_0

Click OK when prompted to run automation.

Notice that external ports have been automatically added for the DDR and Fixed IO once Block Automation has been complete; some of the other default ports are also added to the block.

(a) ZedBoard					   (b) Zybo

Figure A-6. Zynq Block with DDR and Fixed IO ports

In the block diagram, double click on the Zynq block to open the Customization window for the Zynq processing system.

A block diagram of the Zynq should now be open, showing various configurable blocks of the Processing System.

At this stage, the designer can click on various configurable blocks (highlighted in green) and change the system configuration.

Configure I/O Peripherals block to use UART 1 and  I2C 1 peripherals, disabling other unwanted peripherals. Uncheck Timer 0. Enable FCLK_CLK1, the PL fabric clock and set its frequency either to 10.000 MHz for the ZedBoard or to 12.288 MHz for the Zybo.

Select the MIO Configuration tab on the left to open the configuration form and expand I/O Peripheral in the right pane.

Click on the check box of the I2C 1 peripheral. Uncheck USB0, SD 0, ENET 0, GPIO > GPIO MIO as we don’t need them.

Expand the Application Processing Unit group in the Select the MIO Configuration tab and uncheck the Timer 0.

Select the Clock Configuration in the left pane, expand the PL Fabric Clocks entry in the right, and click the check-box of FCLK_CLK1.

Change the Requested Frequency value of FCLK_CLK1 to 10.000 MHz for the ZedBoard or 12.288 MHz for the Zybo.

(a) ZedBoard

(b) Zybo

Figure A-7. Enabling and setting the frequency of FCLK_CLK1

Click OK.

Notice that the Zynq block only shows the necessary ports.

Add the provided I2C-based either zed_audio_ctrl IP for the ZedBoard or zybo_audio_ctrl IP for the Zybo to the IP Catalog

In the Flow Navigator pane, click IP Catalog under Project Manager.

The IP Catalog will open.

Figure A-8. Invoking IP Catalog

Click on the IP Settings button in the IP Catalog.

Figure A-9. Invoking IP Settings

Click on the Add Repository… button. Browse to c:\xup\hls\labs\lab4\ip_repo directory, and click Select.

Notice that the zed_audio_ctrl and the zybo_audio_ctrl entries are displayed in the IP in Selected Repository field.

Figure A-10. Adding IP repository for the provided I2C based cores

Click OK to accept the settings.

ZedBoard: Instantiate zed_audio_ctrl and GPIO with width of 2 bits on channel 1 and width of 1 bit input only on channel 2.

- Zybo: Instantiate zybo_audio_ctrl and GPIO with width of 1 bit output only on channel 1 and width of 1 bit input only on channel 2.

- Run connection automation to connect them.

Click the Add IP button  if the IP Catalog is not open and search for AXI GPIO in the catalog by typing gpi and double-click on the AXI GPIO entry to add an instance.

Click on the Add IP to Block Design button.

Double-click on the added instance and the Re-Customize IP GUI will be displayed.

Change the Channel 1 width to 2 for the ZedBoard or width of 1 output only for the Zybo.

Check the Enable Dual Channel box, set the width to 1 input only, and click OK.

Similarly add an instance of either the zed_audio_ctrl for the ZedBoard or the the zybo_audio_ctrl for the Zybo.

Notice that Design assistance is available. Click on Run Connection Automation, and select /axi_gpio_0/S_AXI

Click OK to connect it to the M_AXI_GP0 interface.

Notice two additional blocks, Proc Sys Reset, and AXI Interconnect have automatically been added to the design.

Similarly, click on Run Connection Automation, and select either /zed_audio_ctrl_0/S_AXI for the ZedBoard or the  /zybo_audio_ctrl_0/S_AXI for the Zybo and click OK.

Make IIC_1, GPIO, FCLK_CLK1, and either zed_audio_ctrl or  zybo_audio_ctrl ports external.

Select the GPIO interface of the axi_gpio_0 instance, right-click on it and select Make External to create an external port. This will create the external port named GPIO and connect it to the peripheral.

Select the GPIO2 interface of the axi_gpio_0 instance, right-click on it and select Make External to create the external port.

Similarly, selecting one port at a time either of the zed_audio_ctrl_0 instance or the zybo_audio_ctrl_0 instance, make them external.

Similarly, make the IIC_1 interface and FCLK_CLK1 port of the processing_system7_0 instance external.

At this stage the design should look like shown below (you may have to click the regenerate [] button).

Figure A-11. Block design after I2C based zed_audio_ctrl core added and connections made for the ZedBoard

Figure A-11. Block design after I2C based zybo_audio_ctrl core added and connections made for the Zybo